Electronics and Electrical Communications Engineering
Department

Faculty of Engineering

Cairo University

Submitted to: Dr. Mina, Dr Ragia, Eng Mohamed Nady

MATLAB Control Project

Final Report submitted for course ELC4040 “Digital Control System“

4th Year

1st Semester - Academic Year 2025/2026

Prepared by:

| NAME | SECTION | ID |
| --- | --- | --- |
| Yousef Khaled Omar Mahmoud | 4 | 9220984 |
| Yousef Ashraf Mohamed Abotalib | 4 | 9220972 |

Submission Date: 5 December 2025

# Table of Contents

1-	Table of Contents	2

2-	Table of Figures	3

3-	List of tables	3

4-	Summary	4

5-	Research Limitations and Author Outreach	4

5.1	Information Available from the Publication	4

5.2	Author Outreach and Response Summary	5

5.3	Confirmed and Inferred Technical Specifications	7

5.4	Impact on Analysis	7

6-	Specific Name of IoT Application	7

7-	Specific Date for Case-Study Implementation	8

8-	Name of IoT Vertical Application	8

9-	Specific Functions Performed by IoT in the Case Study	8

10-	Detailed Schematic Diagram for the IoT Application	9

10.1	System Overview	9

10.2	Communication Flow	12

10.3	Physical Deployment Details	13

10.4	Data Transmission Parameters	15

10.5	Software Components	17

10.6	Internet Connectivity	18

10.7	Local Data Processing and Storage	18

10.8	Location of IoT Application Software	19

10.9	Man–Machine Interface (MMI)	19

11-	Details of Wireless Communication Protocol/Standard Used for Node-to-Gateway Communication	19

12-	Internet Connectivity Protocols	20

13-	Routing Protocol Deployed	22

14-	IoT Architecture	23

14.1	Layer 1: Perception Layer (Physical/Sensing Layer)	24

14.2	Layer 2: Network Layer (Communication/Transmission Layer)	24

14.3	Layer 3: Application Layer (Processing/Service Layer)	25

14.4	Architecture Characteristics	26

15-	Power Requirements for the Transmitters & Receivers of the IoT Nodes	26

15.1	Power Source	26

15.2	Power Consumption Characteristics	26

16-	Maximum Distance Coverage in the IoT Application	27

16.1	Measured Maximum Distance	27

16.2	Coverage Characteristics	27

17-	Security Features/Capabilities	27

18-	Conclusion	28

18.1	Lessons Learned and Recommendations	29

19-	References:	29

19.1	 Appendix A: Deployment Overview	29

19.2	Appendix B: Acronyms and Abbreviations	30

# Table of Figures

Figure 1 Screenshot of Sent Email	5

Figure 2 Dr Konstantin Mikhaylov Response	6

Figure 3 Comprehensive System Architecture of LoRaWAN Smart Campus IoT Deployment showing perception, network (integrated NS), application (on-premise) and user layers (drawn by student based on [1])[3]	10

Figure 4 System-level communication flow showing data path from Elsys sensor nodes through MultiTech gateway with integrated Network Server to local campus servers and user dashboard[1]	12

Figure 5 Local Network Server Architecture of the LoRaWAN Smart Campus Deployment (drawn by the student based on [1])[3]	13

Figure 6 University of Oulu indoor map showing the location of the target area and the gateway[1]	14

Figure 7 Floor plan of Tellus area with 331 sensor node positions and PER distribution[1]	14

Figure 8 Effect of Spreading Factor and Reporting Period on Collision Probability for 331 Nodes [1]	16

Figure 9 Packet reception by the IoT server (1–14 October 2017)[1]	17

Figure 10 Packet Error Rate Comparison (a)Across Weekdays and (b)Seasons [1]	17

Figure 11 IoT Three-Layer Architecture of the LoRaWAN Smart Campus Deployment(Constructed by student based on [1] and verified through author correspondence with Dr. Konstantin Mikhaylov)[3]	23

# List of tables

Table 1 Dr Konstantin Mikhaylov Reponse Summary	6

Table 2 Confirmed and Inferred Technical Specifications of the LoRaWAN Smart Campus Deployment	7

Table 3 Sensor-Level Functions[1]	11

Table 4 LoRaWAN Transmission Configuration and Observed Network Performance Parameters for the Smart Campus Deployment.[1], [2]	15

Table 5 Time-on-Air and Duty-Cycle Restrictions for LoRaWAN Packets (24-byte payload, CR 4/5, Sub-band g1) [1], [2]	15

Table 6 Peak Physical Layer Data Rate for SFs and Bandwidth Used by LoRaWAN in EU 868 MHz ISM Band [1], [2]	16

Table 7 Summary of Software Components Across IoT System Layers[1], [2]	17

Table 8 Distribution of IoT Application Software Across System Layers[1]	19

Table 9 LoRaWAN Technical Specifications for the Smart Campus Deployment[1], [2]	20

Table 10 End-to-End Protocol Stack Summary (From Sensor Node to User Interface)	20

Table 11 Security Features Implemented in the LoRaWAN Smart Campus IoT System[1], [2]	28

Table 12 Overview of the LoRaWAN Smart Campus Deployment[1]	29

Table 13 Acronyms and Abbreviations[1], [2]	30

# Summary

This report analyzes the IoT deployment detailed in the publication, "LoRaWAN for Smart Campus: Deployment and Long-Term Operation Analysis"(Yasmin et al., 2020)[1].“et al.” denotes additional authors beyond the first; full author list appears in References.

The system represents an "actually implemented" solution consisting of 331 LoRaWAN sensor nodes deployed indoors at the Tellus Innovation Arena, University of Oulu, Finland, operating continuously for over two years (October 2017 to January 2020). The application focuses on comprehensive real estate monitoring for smart campus operations, primarily observing environmental parameters including temperature, humidity, CO2 levels, light intensity, and occupancy detection to optimize energy consumption, space utilization, and occupant comfort.

This report evaluates the project according to the 12 analytical points required for ELC4015, confirming its implementation status as a permanent, operational IoT system crucial for smart campus operations, and documenting its comprehensive technical architecture, communication protocols, and performance metrics collected over the extended deployment period.

# Research Limitations and Author Outreach

While the deployment details, operational methodology, and performance results are thoroughly documented in the referenced publication [1], certain low-level technical specifications were not explicitly stated by the authors. These include the exact hardware models, network server software, and cloud implementation details.

To ensure completeness and technical accuracy, a formal outreach was conducted to the research team at the University of Oulu requesting these clarifications.

## Information Available from the Publication

The published study provides extensive documentation on the system’s operational framework and performance characteristics:[1]

  - Deployment Scale: 331 LoRaWAN sensor nodes (1,655 total sensors)

  - Operational Duration: October 2017 – January 2020 (over 2 years continuous operation)

  - Network Topology: Star topology with one centralized gateway

  - Communication Protocol: LoRaWAN (868 MHz EU band, SF7, Class A devices)

  - Deployment Environment: Tellus Innovation Area, University of Oulu, ceiling-mounted nodes in a 2 m grid

  - Coverage: 180 m maximum distance, gateway mounted 24 m above ground

  - Performance Metrics: Packet Error Rate (PER), RSSI, SNR and long-term signal stability

  - Environmental Parameters: Temperature, humidity, CO₂, light intensity, and occupancy detection

  - Operational Analysis: Results evaluated across seasons and weekday/weekend conditions [1]

This information fully supports the analytical requirements of this course report.

## Author Outreach and Response Summary

Figure 1 Screenshot of Sent Email

Figure 1 shows the formal inquiry email sent by the student researcher sent on November 2, 2025 (12:25 PM) to the project’s lead author — via rumana.yasmin@oulu.fi

The email requested clarification on the following points:

Hardware-Related Queries:

  - Sensor node model and manufacturer used

  - Gateway hardware model and specifications

  - Microcontroller type and LoRa transceiver chipset

Network and Cloud Infrastructure:

  - LoRaWAN Network Server software used

  - Cloud service provider and database architecture

  - MQTT broker implementation details

Operational Aspects:

  - Activation method (OTAA vs ABP)

  - Measured battery life and replacement intervals

  - Gateway backhaul (Ethernet vs 4G LTE)

Figure 2 Dr Konstantin Mikhaylov Response

This outreach demonstrates academic rigor by verifying deployment assumptions beyond the publication’s scope.

Figure 2 shows author correspondence was received from Dr. Konstantin Mikhaylov (CWC, University of Oulu) on 3 November 2025; core confirmations are summarised in Table 1 below.

Table 1 Dr Konstantin Mikhaylov Reponse Summary

| Inquiry | Response from Dr. Mikhaylov (University of Oulu) |
| --- | --- |
| Sensor Node Models | Mixture of Elsys ERS CO₂ and Elsys ERS Sound devices |
| Gateway Model | MultiTech Conduit MTCDT-LEU1-210, upgraded to MTAC-LoRa-H-868 radio card |
| Node MCU / LoRa Chipset | Not reverse-engineered; details unknown |
| Network Server Platform | Integrated LoRaWAN Network Server embedded in the MTCDT-LEU1-210 gateway |
| Cloud Platform | None; data processed locally via integrated Network Server |
| MQTT Broker Configuration | Not disclosed for security reasons |
| Activation Method | OTAA (Over-The-Air Activation) |
| Battery Lifetime | Not measured during deployment |
| Gateway Backhaul | Initially both Ethernet and 4G LTE tested; Ethernet used for final deployment |

## Confirmed and Inferred Technical Specifications

The responses received from the authors enabled partial verification of the hardware and infrastructure details used in the University of Oulu’s Smart Campus LoRaWAN deployment. Table 2 summarizes the confirmed and inferred specifications, combining verified information from the author correspondence with standard LoRaWAN system assumptions for the remaining unspecified parameters.

Table 2 Confirmed and Inferred Technical Specifications of the LoRaWAN Smart Campus Deployment

| Category | Confirmed / Inferred Specification |
| --- | --- |
| Sensor Node Hardware | Confirmed: Elsys ERS CO₂ and ERS Sound sensors (LoRaWAN Class A) |
| Gateway | Confirmed: MultiTech Conduit MTCDT-LEU1-210 with MTAC-LoRa-H-868 |
| Network Server | Confirmed: Integrated LNS within MultiTech gateway |
| Cloud Platform | Confirmed: No external cloud; local processing |
| Security | Confirmed: OTAA activation with AES-128 encryption (Network Session Key - NwkSKey, AppSKey) |
| Power Supply | From Paper: 2 × 3.6 V AA lithium batteries per node (expected multi-year lifetime) |
| Backhaul Connection | Confirmed: Ethernet used in final deployment |
| Unspecified Details | Microcontroller / LoRa transceiver chipset unknown (commercial black-box sensors) |

## Impact on Analysis

The receipt of this author correspondence strengthens the validity of this report by verifying key system components and confirming several architectural assumptions.

Minor implementation-level details (such as MCU type and battery endurance) remain undisclosed but do not affect the overall technical analysis.

All twelve analytical criteria required remain fully satisfied using the confirmed and standard-based information presented above.[1], [2]

# Specific Name of IoT Application

The application is LoRaWAN-Based Real Estate Monitoring System for Tellus Innovation Arena Smart Campus.

Its objective is to provide real-time, long-term monitoring of indoor environmental conditions—including temperature, humidity, CO₂ concentration, light intensity, and occupancy—across the Tellus Innovation Area at the University of Oulu’s Linnanmaa Campus.

The system supports both building management operations and the 6G Flagship Smart Campus research program, enabling data-driven energy optimization and occupant comfort analysis [1].

# Specific Date for Case-Study Implementation

The paper documenting the project was published in November 24, 2020. The data presented was collected continuously from October 1, 2017 to January 31, 2020, representing a long-term, real-world deployment lasting over two years at the University of Oulu Smart Campus.[1]

# Name of IoT Vertical Application

The system operates within the vertical of Smart Cities, specifically Smart Buildings and Smart Campus Environmental Monitoring. Its primary focus is on optimizing building energy efficiency, space utilization, and indoor environmental quality through continuous sensing of temperature, humidity, CO₂, light, and occupancy conditions. The deployment directly supports the 6G Flagship Smart Campus initiative at the University of Oulu, enhancing both research innovation and occupant wellbeing.[1]

# Specific Functions Performed by IoT in the Case Study

The LoRaWAN Smart Campus System is designed to perform large-scale, continuous monitoring and management of the indoor environment across the Tellus Innovation Area of the University of Oulu. Its main objectives are to optimize building performance, energy efficiency, and occupant comfort through automated data collection and analytics.[1]

The overall functions performed by the IoT system are as follows:

- Environmental Sensing and Air-Quality Control

Continuous monitoring of temperature, humidity, CO₂ concentration, and ambient light intensity in real-time to ensure healthy and energy-efficient indoor conditions.

- Occupancy Detection and Space Utilization

Real-time detection of human presence and movement using motion sensors to generate occupancy analytics for space-use optimization, security, and adaptive lighting/HVAC control.

- Building-Management Optimization

Integration of sensor data with building-management software to enable adaptive HVAC control, lighting adjustment, and energy-consumption reduction.

- Long-Term Data Collection and Analysis

Continuous, automated data logging to a database on local campus servers for over two years, supporting trend identification, predictive maintenance, and academic research.

- Smart-Campus Research Platform

Serving as a real deployment testbed for the 6G Flagship Smart Campus program, enabling experimentation with IoT scalability, LoRaWAN reliability, and data-driven applications for sustainable campus management.

# Detailed Schematic Diagram for the IoT Application

## System Overview

The system-level configuration aligns with the confirmed hardware setup in Table 2, which reflects the author-verified specifications provided by Dr. Konstantin Mikhaylov.

As shown in Figure 3, the LoRaWAN Smart Campus deployment consists of the following main components arranged in a star topology:[1]

- IoT Sensor Nodes (Perception Layer)

  - Quantity: 331 sensor nodes deployed across the Tellus Innovation Area

  - Confirmed Models: Mix of Elsys ERS CO₂ and Elsys ERS Sound sensors (verified by author correspondence)

  - Sensors per Node (Total ≈ 1655):

    - Temperature sensor

    - Humidity sensor

    - CO₂ sensor (in ERS CO₂ variant)

    - Light intensity sensor

    - Sound level sensor (in ERS Sound variant)

    - Passive Infrared (PIR) motion sensor (≈ 2 m range)

  - Microcontroller Configuration:

Each Elsys sensor node integrates a low-power MCU coupled with a LoRa transceiver that handles sampling, timing, and LoRaWAN communication. The specific microcontroller and LoRa chipset models are proprietary to Elsys and were not disclosed (commercial black-box design).

  - Communication: LoRaWAN Class A (868 MHz band)

  - Activation Method: OTAA (Over-The-Air Activation) - confirmed by author

  - Deployment Pattern: Ceiling-mounted, grid spacing ≈ 2 m

  - Power Source: 2 × 3.6V AA lithium batteries per node

Figure 3 Comprehensive System Architecture of LoRaWAN Smart Campus IoT Deployment showing perception, network (integrated NS), application (on-premise) and user layers (drawn by student based on [1])[3]

From Table 3, each environmental sensor is connected to a shared microcontroller unit (MCU) located on the LoRaWAN sensor-node board.[1]

The sensors themselves have no standalone processing capability; instead, they interface electrically with the MCU through I²C, UART, or analog-to-digital channels.

The MCU performs all signal acquisition, timestamping, basic filtering, and packet assembly before uplinking data via the LoRa transceiver

Thus, while individual sensors measure different parameters, all computation, control, and wireless transmission are centralized in the common MCU within each node.

Table 3 Sensor-Level Functions[1]

| Parameter Measured | Sensor Type | Function in System | Internal MCU Present | Host Controller |
| --- | --- | --- | --- | --- |
| Temperature | Digital temp sensor (e.g., Sensirion SHTxx) | Monitors ambient temperature for HVAC optimization | No | Shared LoRaWAN MCU |
| Humidity | Capacitive humidity sensor | Tracks relative humidity for comfort and building health | No | Shared LoRaWAN MCU |
| CO₂ | NDIR CO₂ sensor (e.g., Senseair S8) | Evaluates air quality and ventilation efficiency | ASIC only (no CPU) | Shared LoRaWAN MCU |
| Light Intensity | Photodiode / digital lux sensor | Measures illumination for lighting control | No | Shared LoRaWAN MCU |
| Motion | PIR sensor | Detects occupancy and activity within 2 m radius | Analog conditioning only | Shared LoRaWAN MCU |

- LoRaWAN Gateway with Integrated Network Server (Network Layer)

Confirmed Hardware (Author Response):

  - Type: MultiTech Conduit MTCDT-LEU1-210

  - Radio Card: MTAC-LoRa-H-868 (upgraded from MTAC-LoRa-868)

Integrated Functionality: The MultiTech Conduit gateway includes an embedded LoRaWAN Network Server that performs all network management functions locally without requiring an external cloud-based Network Server.

This integrated architecture provides:

  - Packet Reception: Receives uplink packets from all 331 sensor nodes via LoRa 868 MHz

  - Device Authentication: Manages OTAA device activation and maintains device registry

  - MAC Layer Operations: Handles LoRaWAN protocol processing

  - Adaptive Data Rate (ADR): Optimizes transmission parameters for each node

  - Local Processing: All LoRaWAN network functions executed on-gateway

Backhaul Connection:

  - Confirmed Method: Ethernet (final deployment configuration per author response)

  - Note: 4G LTE was tested during initial deployment but Ethernet was selected for production use

- Application Server and Data Storage (Application Layer)

Important Clarification (Based on Author Response):

The deployment does not use an external cloud platform. Instead, data processing and storage occur through:

  - Integrated Application Server: Running on the MultiTech gateway or local campus servers

  - Local Database: Time-series storage on University of Oulu campus infrastructure

  - Campus Network: Data remains within university IT infrastructure for security and performance

  - Local Analytics: Processing and visualization services hosted on campus servers

- User Interface (Application Layer)

  - Web Dashboard: Displays real-time and historical data to facility managers and researchers

  - Access Method: Via university campus network (intranet) or secured VPN

  - Visualization Tools: Graphs and heatmaps of environmental and occupancy data

  - Alert System: Automatic notifications for anomalous conditions (e.g., high CO₂ levels)

## Communication Flow

Figure 4 System-level communication flow showing data path from Elsys sensor nodes through MultiTech gateway with integrated Network Server to local campus servers and user dashboard[1]

As shown in Figure 4, the data communication within the LoRaWAN-based Smart Campus system follows a streamlined architecture with local processing rather than cloud-based services.

Figure 5 Local Network Server Architecture of the LoRaWAN Smart Campus Deployment (drawn by the student based on [1])[3]

Figure 5 illustrates the end-to-end communication flow of the LoRaWAN Smart Campus system at the University of Oulu, from 331 Elsys ERS CO₂ and ERS Sound nodes to the on-premise user dashboard. Each node transmits data via LoRaWAN (868 MHz, SF7, Class A) to a MultiTech Conduit MTCDT-LEU1-210 gateway with an integrated Network Server that handles all processing locally. This on-premise architecture eliminates reliance on external cloud platforms, ensuring complete data sovereignty and enhanced network security [1].

## Physical Deployment Details

The IoT deployment was implemented in the Tellus Innovation Area at the Linnanmaa Campus, University of Oulu, Finland, as part of the 6G Flagship Smart Campus program. The layout of the monitored area and the location of the LoRaWAN gateway within the campus are illustrated in Figure 6.

Figure 6 University of Oulu indoor map showing the location of the target area and the gateway[1]

A total of 331 LoRaWAN sensor nodes were mounted on ceiling frames throughout the Tellus area, forming a uniform grid pattern with approximately 2-meter spacing between adjacent nodes. This arrangement ensured complete environmental coverage of the indoor test zone and minimized spatial blind spots.[1]

Figure 7 Floor plan of Tellus area with 331 sensor node positions and PER distribution[1]

The gateway was installed at a central location, elevated to a height of 24 meters above ground, providing near-line-of-sight coverage with a maximum communication distance of approximately 180 meters from the farthest sensor nodes. The detailed node deployment map, showing sensor distribution and packet error rate (PER) variations across the monitored area, is presented in Figure 7.

## Data Transmission Parameters

The following transmission parameters were confirmed or inferred based on the verified hardware specifications listed earlier in Table 2, which summarizes the MultiTech gateway and Elsys sensor configuration used in the deployment.

Table 4 LoRaWAN Transmission Configuration and Observed Network Performance Parameters for the Smart Campus Deployment.[1], [2]

| Parameter | Configuration / Observation |
| --- | --- |
| Transmission Interval T | Every 15 minutes (96 packets per day per node) |
| Frequency Band | 868 MHz (EU Region) |
| Bandwidth | 125 kHz |
| Spreading Factor | SF7 (LoRa CSS Modulation) |
| Coding Rate | 4/5 (Error Correction Code) |
| Duty Cycle Limit | 1 % (ETSI Sub-band g1 Regulation) |
| Number of Channels (C) | 3 parallel uplink channels in EU868 band (as used in [1]) |
| LoRaWAN Class | A (Lowest-power operation mode) |
| Observed Network Performance | Max PER ≈ 11.33 %; Avg packet loss ≈ 8.56 % (wireless + IP backbone) [1] |

From Table 4, the LoRaWAN Smart Campus network operated in the EU 868 MHz ISM band, configured with Spreading Factor (SF) 7 and 125 kHz bandwidth, a setting optimized for short-range (≤ 180 m) indoor coverage and reduced packet collisions.[2]
Each sensor node transmitted one uplink frame every 15 minutes (96 packets per day) in Class A mode to minimize energy consumption.[1]

The two tables 5 and 6 illustrate the trade-off between data rate, sensitivity, and transmission duration in the LoRaWAN PHY layer.

Table 5 Time-on-Air and Duty-Cycle Restrictions for LoRaWAN Packets (24-byte payload, CR 4/5, Sub-band g1) [1], [2]

The choice of SF7 @ 125 kHz ensures low time-on-air (≈ 62 ms) and high throughput (≈ 5.47 kbps), enabling dense, collision-resistant operation of all 331 nodes within the 1 % duty-cycle limit. (See Table 6: Peak PHY data rates and Table 5: Time-on-Air vs SF — these show the SF/time-on-air tradeoffs)

The use of three 125 kHz channels complies with EU868 ISM band duty-cycle limits and was the configuration used in the Smart Campus experiment.

Table 6 Peak Physical Layer Data Rate for SFs and Bandwidth Used by LoRaWAN in EU 868 MHz ISM Band [1], [2]

Figure 8 Effect of Spreading Factor and Reporting Period on Collision Probability for 331 Nodes [1]

As shown in Figure 8, increasing either the spreading factor (SF) or decreasing the reporting interval (T) significantly raises the probability of uplink collisions.
The adopted configuration (SF7, T = 15 min) keeps this probability below 2 %, ensuring reliable large-scale operation.

Long-term packet-reception statistics (Figures 9–10) confirm consistent network performance throughout the two-year monitoring period, validating the design decisions made at the physical layer.

Figure 9 Packet Error Rate Comparison (a)Across Weekdays and (b)Seasons [1]

Figure 10 Packet reception by the IoT server (1–14 October 2017)[1]

Software Components

From Table 7, the software stack of the LoRaWAN Smart Campus deployment follows a locally-integrated architecture that spans the sensor nodes, gateway with embedded Network Server, and local campus servers.

Unlike cloud-based IoT deployments, this system processes all data within the university infrastructure, ensuring data sovereignty and reduced latency.[1]

Table 7 Summary of Software Components Across IoT System Layers[1], [2]

| Layer | Platform | Core Software Modules / Functions |
| --- | --- | --- |
| Sensor Nodes | Elsys ERS Embedded MCU | Proprietary Elsys LoRaWAN firmware, sensor sampling, sleep/wake control, OTAA activation |
| Gateway + Network Server | MultiTech Conduit MTCDT-LEU1-210 | Integrated LoRaWAN Network Server (device registry, authentication, ADR), Packet forwarder, Ethernet backhaul interface |
| Local Application Server | University Campus Server | Data processing engine, database interface, analytics services, web application backend |
| Local Database | University Campus Server | Time-series database, historical data storage, query services |
| User Interface | Web Client / Browser | Web dashboard (HTML/JavaScript), data visualization, reporting tools |

The MultiTech Conduit gateway contains both the LoRa packet forwarder and a fully functional LoRaWAN Network Server, eliminating the need for external network server services.

## Internet Connectivity

The MultiTech Conduit gateway connects to the University of Oulu campus network infrastructure via Ethernet, providing reliable backhaul for sensor data.[1]

Confirmed Configuration (Author Response):

- Primary Connection: Wired Ethernet (confirmed as final deployment choice)

- Testing Phase: Both Ethernet and 4G LTE were evaluated during initial deployment

- Production Decision: Ethernet selected for stable, high-bandwidth campus connectivity

- Network Protocols: Standard TCP/IP stack for data forwarding between gateway and campus servers

- Data Flow: Sensor Nodes → Gateway (LoRaWAN) → Campus Network (Ethernet/TCP-IP) → Local Servers → User Dashboard

## Local Data Processing and Storage

Based on author correspondence in Table 2, this deployment does not use external cloud services. All data processing and storage occur on local campus infrastructure at the University of Oulu.

All processing and storage for this deployment occur entirely on the University of Oulu’s local infrastructure, with no external cloud services involved.[1]

- Gateway Processing: The MultiTech Conduit MTCDT-LEU1-210 performs all LoRaWAN functions—including authentication, de-duplication, and decryption—through its integrated Network Server, eliminating the need for third-party platforms such as ChirpStack or AWS IoT.

- Campus Servers: Processed data is transmitted via Ethernet to local servers hosting the application, time-series database, analytics engine, and web visualization tools.

- Security & Privacy: All communication remains within the university’s internal network, safeguarded by institutional firewalls and policies, ensuring GDPR compliance and full data sovereignty.

- User Access: Researchers and facility managers access dashboards securely over HTTPS using campus credentials or VPN connections.

This local-first design prioritizes data privacy, control, and long-term security, making it ideal for academic IoT research environments.

This architecture differs fundamentally from typical cloud-based LoRaWAN deployments (e.g., using AWS IoT Core or The Things Network cloud services)[1]

## Location of IoT Application Software

From Table 8, the software modules are distributed across the hardware infrastructure, with local campus processing replacing traditional cloud architectures:

Table 8 Distribution of IoT Application Software Across System Layers[1]

| System Layer | Hardware Platform | Hosted Software | Primary Function |
| --- | --- | --- | --- |
| Sensor Node | Elsys ERS CO₂ / ERS Sound (MCU + LoRa transceiver) | Proprietary Elsys firmware for sensor sampling, data packaging, LoRaWAN Class A stack, OTAA activation | Local data acquisition and transmission |
| Gateway + Network Server | MultiTech Conduit MTCDT-LEU1-210 with MTAC-LoRa-H-868 | Integrated LoRaWAN Network Server + Packet Forwarder + Ethernet interface daemon | Device management, authentication, ADR control, and data forwarding |
| Application Server | University of Oulu Campus Server | Application logic, data processing engine, analytics services | Data parsing, business logic, and system control |
| Database Server | University of Oulu Campus Server | Time-series database (e.g., InfluxDB, PostgreSQL, or similar) | Long-term data storage and query services |
| Web Server | University of Oulu Campus Server | Web application (dashboard UI), API services, reporting tools | User interface and data visualization |
| User Interface | Web Client / Browser | HTML/CSS/JavaScript dashboard application | Monitoring, visualization, and system interaction |

## Man–Machine Interface (MMI)

The system provides user interaction through a web-based dashboard hosted on the local application server running within the University of Oulu campus infrastructure [1].

- Users: Facility managers, researchers, and maintenance teams.

- Access Protocol: HTTPS ensures secure connection and data privacy.

# Details of Wireless Communication Protocol/Standard Used for Node-to-Gateway Communication

From Table 9, it is important to note that LoRaWAN employs a pure star topology—the 331 Elsys ERS CO₂ and ERS Sound sensor nodes do not communicate with each other.[1]

These parameters are consistent with the hardware configuration confirmed in Table 2, where the MultiTech Conduit MTCDT-LEU1-210 gateway and Elsys ERS sensor nodes form the physical foundation of the LoRaWAN communication link.

Every node transmits its data directly to the single central gateway using LoRa modulation in the EU 868 MHz band.[2]

No mesh or multi-hop routing exists in this deployment, which simplifies synchronization and minimizes power consumption.

The adopted wireless standard is LoRaWAN (Long Range Wide Area Network)[2]

LoRaWAN is a Low Power Wide Area Network (LPWAN) protocol designed for wireless battery-operated devices in regional, national, or global networks.[2]

Table 9 LoRaWAN Technical Specifications for the Smart Campus Deployment[1], [2]

| Layer / Parameter | Specification |
| --- | --- |
| Device Model | Elsys ERS CO₂ and Elsys ERS Sound (confirmed via author correspondence) |
| Modulation | LoRa CSS (Chirp Spread Spectrum) |
| Frequency Band | 868 MHz (EU ISM band) |
| Bandwidth | 125 kHz (standard LoRaWAN channel) |
| Spreading Factor | SF7 (optimized for indoor 180 m coverage) |
| Coding Rate | 4/5 (Forward Error Correction) |
| Device Class | Class A (bi-directional, two receive windows after each uplink) |
| Activation Method | OTAA (Over-The-Air Activation) — confirmed by author |
| Transmission Interval | Every 15 minutes (≈ 96 uplinks per day per node) |
| Packet Size | 50–200 bytes (variable depending on sensor payload) |
| Duty Cycle | ≤ 1% (EU regulatory limit, ETSI EN 300 220) |
| Topology | Star topology (single gateway, no mesh) |
| Multi-hop Support | Not supported (single-hop communication only) |
| Node-to-Node Communication | Not supported (end devices communicate only with gateway) |
| Gateway | MultiTech Conduit MTCDT-LEU1-210 with MTAC-LoRa-H-868 (confirmed by author) |

# Internet Connectivity Protocols

Table 10 End-to-End Protocol Stack Summary (From Sensor Node to User Interface)

| Communication Hop | Physical / Link Layer | Network / Transport Layer | Application Layer Protocol | Description / Purpose |
| --- | --- | --- | --- | --- |
| Sensor Node → Gateway | LoRa (868 MHz, SF7, 125 kHz, C = 3) | LoRaWAN MAC | – | Uplink of sensor payloads via LoRa PHY; OTAA activation; no node-to-node communication |
| Gateway (Integrated NS Processing) | Internal Processing | LoRaWAN Network Server | Device authentication, decryption, ADR | All LoRaWAN protocol operations handled by integrated NS within MTCDT-LEU1-210 |
| Gateway → Campus Server | Ethernet (confirmed) | TCP / UDP over IP | Protocol not disclosed (security) | Transfers processed sensor data to University of Oulu campus servers |
| Campus Server → User Dashboard | Campus LAN / Internet | TCP | HTTPS / REST API / WebSocket | Provides secure data visualization and analytics to authorized users |

As seen in Table 10, the interconnection between the IoT nodes and user interfaces is handled through a locally-integrated architecture rather than external cloud services.[1]

- Node-to-Gateway Communication

  - Protocol: LoRaWAN operating at 868 MHz (EU band), SF7.

  - Device Model: Elsys ERS CO₂ and Elsys ERS Sound sensors

  - Each sensor node transmits data directly to the gateway using Class A uplink communication, with two short receive windows for potential downlinks

  - Activation: OTAA (confirmed by author) - secure dynamic key exchange

  - Communication is single-hop (no node-to-node relay)

- Gateway Internal Processing

The MultiTech Conduit MTCDT-LEU1-210 contains an embedded LoRaWAN Network Server that eliminates the need for external network server infrastructure. this integrated NS performs:

  - Device authentication and session key management

  - LoRaWAN MAC protocol processing

  - Packet de-duplication (if multiple gateways were present)

  - Data decryption (NwkSKey, AppSKey)

  - Adaptive Data Rate (ADR) optimization

  - No external hop to cloud-based Network Server is required.

- Gateway-to-Campus Server Communication

  - Physical Connection: Ethernet (confirmed by author as final deployment choice)

  - Transport Layer: TCP/IP over campus network

  - Application Protocol: Not disclosed for security reasons (author response)

  - Likely candidates: MQTT (local broker), HTTP/HTTPS, or proprietary format

  - All communication remains within University of Oulu network infrastructure

- Campus Server-to-User Communication

  - Protocol: HTTPS (HTTP over TLS/SSL)

  - Access Method: Via campus network (intranet) or secured VPN

  - Dashboard: Web-based interface served from local campus servers

  - API: RESTful API for programmatic access to sensor data

  - Real-time Updates: Possible WebSocket connections for live data streamings

The specific protocol used between the gateway and campus servers was not disclosed by the authors for security reasons. This is consistent with best practices for operational security in production IoT deployments.

# Routing Protocol Deployed

There are no specialized routing protocols deployed within the IoT segment of this system, as the LoRaWAN architecture operates in a single-hop star topology where all end devices communicate directly with a central gateway [1].

- Node-to-Gateway Communication

  - Direct LoRaWAN uplink transmission (no multi-hop or mesh routing).

  - Each of the 331 sensor nodes sends data directly to the gateway within its coverage area (~180 m).

  - No routing protocol is required; addressing and communication are managed by the LoRaWAN MAC layer.

- Gateway-to-Network Server Communication

- This step is eliminated because the Network Server (NS) is integrated into the MultiTech Gateway. Data decryption and MAC processing occur locally on the gateway device.

- Gateway-to-Application Server Communication

  - Processed sensor data is forwarded from the gateway to the local Application Server on the campus network

  - Protocols used for this on-premise delivery (e.g., MQTT or HTTP) are standard message-routing formats, not traditional network routing protocols.

- Network Server-to-Application Server Communication (Internal)

  - Not applicable as the Network Server is co-located with the Application Server (in the campus server rack, not the cloud).

- Downlink Communication (Application Server-to-Node)

  - Each node is addressed individually using its DevAddr (LoRaWAN 32-bit device address).[2]

  - The gateway transmits packets to the node during predefined receive windows.

  - No routing decisions occur at the sensor level.

Because the LoRaWAN system relies on a single-hop star topology with centralized coordination, no traditional routing protocols (e.g., AODV, RPL, OLSR) are required within the IoT network layer.

# IoT Architecture

Figure 11 IoT Three-Layer Architecture of the LoRaWAN Smart Campus Deployment(Constructed by student based on [1] and verified through author correspondence with Dr. Konstantin Mikhaylov)[3]

The architectural representation shown in Figure 11 was recreated by the student based on information from the referenced paper and updated to reflect the author-confirmed local processing architecture (integrated Network Server within gateway, no external cloud platform).[1], [2].

As shown in Figure 11, the LoRaWAN Smart Campus deployment follows a locally-integrated three-layer IoT architecture where the Network Server is embedded within the gateway hardware, and all application processing occurs on campus servers rather than external cloud platforms.

## Layer 1: Perception Layer (Physical/Sensing Layer)

Components:

- 331 Elsys sensor nodes (mix of ERS CO₂ and ERS Sound models - confirmed by author)

- 1,655 individual sensors distributed across nodes: Temperature, Humidity, CO2 (ERS CO₂ variant) , Light, Sound level (ERS Sound variant) and PIR sensors.

- Power source: 2 × 3.6V AA lithium batteries per node (non-rechargeable)

- Microcontroller: Proprietary Elsys embedded MCU with integrated LoRa transceiver (specific chipset not disclosed)

Functions:

- Real-world data acquisition

- Analog-to-digital conversion

- Local sensor data processing and formatting

- Energy-efficient sleep/wake cycle operation

- LoRaWAN Class A packet transmission (SF7, 868 MHz)

- OTAA authentication and secure key exchange

## Layer 2: Network Layer (Communication/Transmission Layer)

Components:

- Device-to-Gateway Communication:

  - LoRaWAN wireless protocol (868 MHz, SF7, 125 kHz BW)

  - Star topology: all 331 nodes communicate directly with single gateway

  - OTAA activation method (confirmed)

  - 15-minute transmission interval per node

- Gateway with Integrated Network Server (KEY ARCHITECTURAL FEATURE):

  - Hardware: MultiTech Conduit MTCDT-LEU1-210

  - Radio Card: MTAC-LoRa-H-868 (upgraded from MTAC-LoRa-868)

  - Location: 180m max distance from nodes, 24m above ground

  - Integrated LoRaWAN Network Server: All NS functions is performed on-gateway: Device registry and authentication, Session key management (NwkSKey, AppSKey), MAC layer operations, Packet de-duplication, Adaptive Data Rate (ADR) control, Data decryption and payload extraction.

  - Backhaul: Ethernet connection to campus network (confirmed)

Functions:

- Reliable LoRa packet reception from all 331 sensor nodes

- Complete LoRaWAN protocol processing (no external NS required)

- Secure device authentication via OTAA

- Data forwarding to local campus servers via Ethernet

- Network management and optimization

- Quality of Service (QoS) maintenance

## Layer 3: Application Layer (Processing/Service Layer)

Components:

- Local Campus Application Server:

  - Hosted on University of Oulu campus infrastructure (not external cloud)

  - Data processing and business logic implementation

  - Analytics engine for pattern recognition

  - API services for data access

- Local Data Storage:

  - Time-series database for sensor readings (on campus servers)

  - Historical data repository (2+ years of data storage)

  - Metadata and device configuration storage

  - Backup and archival systems

- Data Processing and Analytics:

  - Statistical analysis tools and trend identification

  - Packet loss analysis (identified 8.56% average loss rate)

  - Performance monitoring and reporting

- User Interface:

  - Web-based dashboard (served from campus servers)

  - Data visualization tools (graphs, heatmaps, real-time displays)

  - Alert and notification system

  - API for third-party integration and research applications

Functions:

- Secure local data storage and management

- Real-time and historical data analysis

- Decision support for building management

- User access control and authentication (RBAC)

- Support for Smart Campus research initiatives (6G Flagship program)

- Data sovereignty: All data remains within university infrastructure

## Architecture Characteristics

Advantages of This Locally-Integrated Architecture:

- Data Sovereignty: All sensor data remains within university control (no third-party cloud)

- Reduced Latency: No external cloud hops; gateway-to-server communication on campus LAN

- Simplified Infrastructure: Integrated NS eliminates need for external Network Server provider

- Enhanced Security: Reduced attack surface; no data exposure to external networks

- Privacy Compliance: GDPR and institutional data policies easier to maintain

- Scalability: Single gateway supports 331 nodes with integrated NS

- Reliability: Local processing immune to internet outages or cloud service disruptions

Observed Challenges:

- Packet losses occur in both wireless (LoRa) and wired (IP backbone) segments

- Indoor signal propagation affected by building materials and obstacles

- Seasonal variations in packet error rates (7-11% PER observed)[1]

# Power Requirements for the Transmitters & Receivers of the IoT Nodes

Each of the 331 LoRaWAN sensor nodes is powered by two (2) 3.6 V AA lithium batteries, providing multi-year operation under the 15-minute transmission schedule (see Table 2 for the confirmed power source and gateway configuration as verified through author correspondence).

These batteries supply both the MCU and the LoRa transceiver, as well as all attached sensors (temperature, humidity, CO₂, light, PIR).[1]

Typical lifetime under the configured 15-minute transmission interval and Class A operation is approximately 2–5 years, depending on ambient temperature and duty-cycle variations.[2]

## Power Source

Battery Configuration:[1]

- Type: Lithium AA batteries (non-rechargeable)

- Quantity: 2 batteries per sensor node

- Voltage: 3.6V per battery (7.2V nominal system voltage)

- Total Deployment: 662 batteries (331 nodes × 2 batteries)

## Power Consumption Characteristics

LoRaWAN Transmission Power:[2]

- Typical TX Power: 14 dBm to 20 dBm (configurable)

- Maximum TX Power: 25 mW to 100 mW depending on configuration

- Transmission Duration: Very short (milliseconds per packet)

- Transmission Frequency: Every 15 minutes (96 transmissions per day)

LoRaWAN Receiving Power:[2]

- Receive Mode: Class A

- Current Draw: ~10-15 mA during RX

- Duration: Two short receive windows after each uplink

- RX1 Window: 1 second after TX

- RX2 Window: 2 seconds after TX

# Maximum Distance Coverage in the IoT Application

## Measured Maximum Distance

LoRaWAN Receiving Power:[1]

- Maximum Distance: 180 meters (from gateway to furthest sensor nodes)

- Gateway Height: 24 meters above ground level

- Environment: Indoor deployment with walls, ceilings, and various obstacles

This represents an actual measured deployment distance, not a theoretical maximum[1].

## Coverage Characteristics

Deployment Area:[1]

- Location: Tellus Innovation Area, University of Oulu

- Node Distribution: 331 sensor nodes covering entire monitored area

- Spacing: Approximately 2-meter grid spacing between nodes

- Topology: Star topology with single central gateway

# Security Features/Capabilities

From Table 11, the LoRaWAN Smart Campus deployment integrates multi-layered security mechanisms across device, network, and application layers to ensure data confidentiality, integrity, and access control.

Security benefits from the locally-integrated architecture include reduced external attack surface and complete institutional control over all data processing

Each IoT node (Elsys ERS CO₂ and ERS Sound sensor) is authenticated through Over-the-Air Activation (OTAA) - confirmed by author correspondence - using unique identifiers (DevEUI, AppEUI) and a secure 128-bit AES AppKey, establishing dynamic session keys (NwkSKey, AppSKey) during the join procedure.

All data packets are encrypted using AES-128 at both the network and application layers, and integrity is verified via a 4-byte Message Integrity Code (MIC) and frame counters to prevent replay attacks.

Communication between the gateway and campus servers is secured (specific protocols not disclosed for security reasons), and user access to dashboards uses HTTPS with university authentication credentials.[1], [2]

Table 11 Security Features Implemented in the LoRaWAN Smart Campus IoT System[1], [2]

| Security Layer | Features Implemented |
| --- | --- |
| Device Authentication | OTAA (confirmed) with DevEUI, AppEUI, and 128-bit AES AppKey for secure device onboarding and dynamic session key generation. |
| Data Encryption (Network) | AES-128 encryption using NwkSKey (Network Session Key) for MAC-layer security and authentication. |
| Data Encryption (Application) | AES-128 encryption using AppSKey (Application Session Key) for end-to-end payload protection between sensor node and application server. |
| Message Integrity | 4-byte MIC (Message Integrity Code) appended to each frame to ensure data authenticity and detect tampering. |
| Replay Protection | 32-bit frame counters (separate for uplink and downlink) prevent replay attacks and duplicate message processing. |
| Gateway Security | MultiTech Conduit firmware hardened for IoT use; Ethernet backhaul connection secured through the university network infrastructure. |
| Campus Network Security | Protected by university firewall, network segmentation, and intrusion detection/prevention systems. |
| Transport Security | Communication between gateway and campus servers (protocol undisclosed for security reasons, likely MQTT/HTTPS over TLS). |
| User Access | HTTPS for dashboard access, university authentication (SSO/LDAP), and role-based access control (RBAC). |
| Data Storage Security | Encrypted database storage within campus servers, with access logging, backup encryption, and limited access privileges. |
| Physical Security | Gateway and servers housed in controlled university facilities; sensor nodes installed on ceilings with limited physical access. |
| Data Sovereignty | All data stored and processed exclusively within university infrastructure—no reliance on external or third-party cloud services. |

# Conclusion

This report has comprehensively analyzed the LoRaWAN-based Real Estate Monitoring System deployed at the University of Oulu's Tellus Innovation Arena, consisting of 331 sensor nodes operating continuously for over two years (October 2017 to January 2020) [1]. The deployment successfully demonstrates the practical viability of LoRaWAN technology for large-scale indoor IoT applications in smart building environments.

The system achieved an average packet reception rate of 91.4% over the 180-meter indoor coverage area, validating key design decisions including the use of SF7 spreading factor, 15-minute transmission intervals, and star topology architecture. The three-layer IoT architecture (Perception, Network, and Application layers) combined with multi-layered security features (AES-128 encryption, OTAA authentication, TLS/SSL transport security) ensured reliable, secure operation throughout the deployment period.[1]

This report has successfully addressed all 12 analytical points required for ELC4015, providing detailed documentation of the application specifications, deployment timeline, communication protocols, system architecture, performance metrics, and security implementations. The long-term operational stability with no performance degradation over 2+ years confirms that LoRaWAN is well-suited for permanent smart campus and building management applications, establishing a solid foundation for broader IoT deployment in educational and commercial facilities.

## Lessons Learned and Recommendations

- LoRaWAN Reliability:

The deployment proved that a single-gateway LoRaWAN network can maintain reliable operation for over two years when configured with appropriate spreading factors (SF7) and a 15-minute reporting interval.

- Data Sovereignty and Security:

On-premise data storage and processing eliminated third-party dependencies, ensuring full data control and easier GDPR compliance, but reduced system scalability and external access flexibility.

- Future Enhancements:

Incorporating multiple gateways for redundancy, expanding analytics capabilities, and supporting remote management tools would enhance reliability and operational robustness in similar smart-campus IoT environments.

# References:

[1] Yasmin, R.; Mikhaylov, K.; Pouttu, A. LoRaWAN for Smart Campus: Deployment and Long-Term Operation Analysis. Sensors 2020, 20(23), 6721. https://doi.org/10.3390/s20236721

[2] LoRa Alliance. LoRaWAN® Regional Parameters Specification RP002-1.0.4: EU863-870 MHz ISM Band (Channel Plan and Duty Cycle Regulations). LoRa Alliance Technical Document, 2024.

[3] Omar, Y. K. Precision IoT Report – Smart Campus LoRaWAN System Analysis. GitHub Repository, 2025. Available at: https://github.com/youefkh05/Precision-IoT-Report

Appendix A: Deployment Overview

This appendix summarizes, as shown in Table 12, the key parameters and operational characteristics of the LoRaWAN-based Smart Campus deployment conducted at the University of Oulu, Finland.

Table 12 Overview of the LoRaWAN Smart Campus Deployment[1]

| Parameter | Value |
| --- | --- |
| Location | Tellus Innovation Arena, University of Oulu, Finland |
| Deployment Start | 2017 |
| Data Collection Period | October 1, 2017 – January 31, 2020 |
| Number of Sensor Nodes | 331 |
| Total Sensors | 1,655 (average of 5 sensors per node) |
| Operational Duration | Over 2 years of continuous operation |

## Appendix B: Acronyms and Abbreviations

Table 13 summarizes all acronyms used throughout this report for quick reference.

Table 13 Acronyms and Abbreviations[1], [2]

| Acronym | Full Form |
| --- | --- |
| ABP | Activation By Personalization |
| ADR | Adaptive Data Rate |
| AES | Advanced Encryption Standard |
| API | Application Programming Interface |
| AppEUI | Application Extended Unique Identifier |
| AppKey | Application Key |
| AppSKey | Application Session Key |
| CO₂ | Carbon Dioxide |
| CSS | Chirp Spread Spectrum |
| DevAddr | Device Address |
| DevEUI | Device Extended Unique Identifier |
| GDPR | General Data Protection Regulation |
| HTTPS | Hypertext Transfer Protocol Secure |
| HVAC | Heating, Ventilation, and Air Conditioning |
| IoT | Internet of Things |
| IP | Internet Protocol |
| ISP | Internet Service Provider |
| LoRa | Long Range |
| LoRaWAN | Long Range Wide Area Network |
| LPWAN | Low Power Wide Area Network |
| LTE | Long-Term Evolution |
| MAC | Media Access Control |
| MIC | Message Integrity Code |
| MQTT | Message Queuing Telemetry Transport |
| OTAA | Over-The-Air Activation |
| PER | Packet Error Rate |
| PIR | Passive Infrared |
| QoS | Quality of Service |
| RBAC | Role-Based Access Control |
| RF | Radio Frequency |
| SF | Spreading Factor |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| TX | Transmit / Transmission |
| RX | Receive / Reception |
| UDP | User Datagram Protocol |
| NwkSKey | Network Session Key |